B.M.S. College of Engineering, Bangalore – 19

(Autonomous & Affiliated College under VTU)

Department of Electronics & Communication Engineering

| Course Title | ARM Processor & Programming LAB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Course Code | 21ES4CCAPP | Credits | 4 | L-T-P | 3:0:1 |

Course Objectives:

The student should be made to:

- Learn the working of ARM processor

- Understand the Building Blocks of Embedded Systems

- Learn the concept of memory map and memory interface

- Write programs to interface memory, I / Os with processor

- Study the interrupt performance.

Lab requisites:

Software and hardware tools used for ARM PROGRAMMING:

- Keil µVision4 IDE (ASSEMBLY PROGRAMS – SIMULATION - PART - A)

- ARM7TDMI - LPC 2148 EVALUATION BOARD FOR INTERFACING EXPERIMENTS (PART - B)

PART-A EXPERIMENTS USING Keil µVision4 IDE (SIMULATION)

- Add a series of 16-bit numbers stored in sequential location in the memory (called Table) and store the result in memory.

- Add two 64-bit numbers and store the result in a memory location.

- Sum of first 10 integer numbers.

- Multiply two 16-bit binary numbers.

- Find the factorial of a given number.

- Divide an 8-bit variable into two 4-bit nibbles and store one nibble in each byte of a 16-bit variable. Store the disassembled byte in memory location (pointed by result).

- Compare 2 values stored in memory location and store the higher value in a memory location (pointed by result).

- Find the largest in a series of numbers stored in memory.

PART A - ASSEMBLY PROGRAMS EXECUTION STEPS - Keil µVision4 IDE

- Open Keil µVision4 IDE software by double clicking on Keil µVision4 icon:

- Go to “Project”, select “New µvision Project” and save the project:

- Once the project is created, select the target device - select LPC2148, click on OK

- A dialog box with “copy startup. s to project folder and add file to project?” will pop up, select NO.

- Thus, the target is created along with source group1 as shown below:

- Open the text editor. Write the program and save the program with the extension .asm

- Once the program is saved, the program file must be added to the target.

Right click on the “Source Group 1”, click on Add existing files to group “source group 1” and then select the assembly program and click on Add, Close.

- Once the program is added to the source group, right click, select Build target or press F7

- Once the target is built, it displays the information with any errors or zero errors.

- If no errors, we can proceed for debug, select start /stop debugging option.

- Then select Run option from the debug menu or can use the key F5.

If F5 key - continuous run

If F11key - step one line

If F10key – step over line

- The results obtained will be verified in the respective registers and in memory window as shown below:

Memory window:

Registers window:

**********************************************************************

Instruction set (for reference)

    - Write an ALP to Add a series of 16-bit numbers stored in sequential location in the memory (called Table) and store the result in memory.

    - Write an ALP to Add two 64-bit numbers and store the result in a memory location.

    - Write an ALP to find sum of first 10 integer numbers.

AREA SUM1, CODE, READONLY

START

MOV R1, #10;

MOV R2, #00;

LOOP

ADDS R2, R2, R1;

SUBS R1, R1, #01;

BNE LOOP;

L       B L

NOP

END

    - Write an ALP to Multiply two 16-bit binary numbers.

    - Find the factorial of a given number

AREA FACTORIAL, CODE, READONLY

ENTRY                                                                        ;Mark first instruction to execute

START

MOV r0, #7                                                ; STORE FACTORIAL NUMBER IN R0

MOV r1, r0                                                 ; MOVE THE SAME NUMBER IN R1 FACT

FACT SUBS r1, r1, #1                                         ; SUBTRACTION

CMP r1, #1                                                ; COMPARISON

STOP 	BEQ STOP

MUL r3, r0, r1                                        ; MULTIPLICATION

MOV r0, r3                                               ; Result

BNE FACT                                                 ; BRANCH TO THE LOOP IF NOT EQUAL STOP

NOP

NOP

NOP

END                                                            ;Mark end of file

    - Divide an 8-bit variable into two 4-bit nibbles and store one nibble in each byte of a 16-bit variable. Store the disassembled byte in memory location (pointed by result).

    - Compare 2 values stored in memory location and store the higher value in a memory location (pointed by result).

    - Find the largest in a series of numbers stored in memory.

******************************************************************************